<< SPECIFIER NOTE:

<< Specifier Notes are shown in Red throughout this document to assist you with modifying its content to suit your project. >>

SHOWER PANS / SHOWER RECEPTORS

SHOWER WALL PANELS

SHOWER TRIM MOLDINGS

SHOWER ACCESSORIES

Tower Industries, Inc.

2101 Ninth St. SW, Massillon OH  44647

330.837.2216

www.TowerSurfaces.com

PART 1 – GENERAL

1.1 RELATED SECTIONS

- Related work specified in other sections:

01 33 00	“Submittal Procedures”

06 10 00 	“Rough Carpentry”

06 20 00 	“Finish Carpentry”

07 92 00 	“Joint Sealants”

09 72 00	“Wall Coverings”

22 42 00 	“Commercial Plumbing Fixtures”

22 43 00	“Healthcare Plumbing Fixtures”

1.2 REFERENCES

- Codes and Standards referenced in this section refer to the latest edition thereof.

- Applicable Standards referenced herein:

ASTM: American Society for Testing and Materials

ANSI: American National Standards Institute

CSA: Canadian Standard Association

IAPMO: International Association of Plumbing and Mechanical Officials

NSF: National Sanitation Foundation

1.3 SUMMARY

- Section includes:

Meridian Solid Surface® material, as indicated and specified

- Work Specified in this Section:

One-piece Standard Shower Pans/Shower Receptors

Custom Shower Pans/Shower Receptors

Custom Shower Thresholds

Shower and Tub Wall Panels

Shower Accessories and Trim Moldings

1.4 DEFINITION

- Solid surface is a homogeneous thermoset polymer compound blended with pigments and/or mineral fillers and filled with alumina trihydrate. Material is non-porous and homogenous, with uniform color and pattern throughout the entire thickness.

  - SUBMITTALS

- Shop Drawings

Installer shall provide detailed shop drawings indicating all details for installation.

- Submit samples of full range of available colors for color selection

- Care & Maintenance Data

Provide manufacturer’s care and maintenance instructions

  - DELIVERY, STORAGE AND HANDLING

- Transport and deliver all items in appropriate packaging to prevent damage.

- Handle materials to prevent damage to adjacent surfaces and previous work.

- Store materials as per manufacturer’s recommendations

- Carry all shower wall panels vertically so as to reduce the chance of breakage during transport.

1.7 QUALITY ASSURANCE

- Source Limitations

Where solid surface shower bases, solid surface wall panels, or other solid surface shower components are indicated on drawings, the material shall be Meridian Solid Surface®, manufactured by Tower Industries, Inc., 2101 Ninth St. SW, Massillon, OH  44647. Phone 330.837.2216

- Certifications

Full-size sample of shower receptor shall be tested and certified annually by an independent third-party laboratory for compliance with the harmonized CSA B45.5/IAPMO Z124 standard.

- Installation Qualifications

Meridian Solid Surface® Shower products shall be installed in accordance with published Meridian shower product installation procedures available for download at www.TowerSurfaces.com.

1.8 WARRANTY

- Furnish the manufacturer’s Limited Fifteen-Year Commercial Warranty covering defects in material and workmanship. Term of warranty to commence upon date product is delivered to the jobsite.

PART 2 - PRODUCTS

2.1 MANUFACTURER / BASIS OF DESIGN

Tower Industries, Inc.

2101 Ninth St. SW, Massillon OH  44647

330.837.2216

www.TowerSurfaces.com

2.2 MATERIALS

- Solid Polymer Fabrications

  - Solid surface material shall be Meridian Solid Surface®, manufactured by Tower Industries, Inc., 2101 Ninth St. SW, Massillon, OH  44647. Phone 330.837.2216

  - Solid surface material shall be a homogeneous thermoset polymer compound blended with pigments and/or mineral fillers and filled with alumina trihydrate. Materials shall be non-porous and homogenous, with uniform color and pattern throughout the entire thickness. Materials shall be 100% repairable.

  - Solid surface material shall meet or exceed all performance requirements listed in Section F below.

  - All products shall carry a Limited Fifteen-Year Commercial Warranty covering defects in material and workmanship.

- Products

  - Standard, One-Piece Shower Pans/Shower Receptors

    - Standard shower pans shall be Meridian Solid Surface®, manufactured by Tower Industries, Inc., 2101 Ninth St. SW, Massillon, OH  44647. Phone 330.837.2216

    - Standard shower pans shall be manufactured from solid cast polymer and have fixed drain locations (or no drain orifices for field-drilling), coved side walls, radius transitions at curbs and thresholds, a minimum 1” wide panel platform with a minimum 5° slope and a 1” minimum water dam.

    - Standard shower pans shall be manufactured as a one-piece, seamless, modular unit with no joints in the shower floors, curbs, or thresholds.

    - Shower pans shall have a pebble-textured, non-skid floor pitched toward the drain at a minimum 0.6° slope (1/8” per foot)

    - Shower pans shall have a non-slip co-efficient rating of .30 or greater as registered by the ASTM F462 slip resistance test method.

    - Shower Pans shall be a minimum of 9/16” thick at the thinnest point of the drain orifice

<< SPECIFIER NOTE:

<< Choose section vii, viii, or ix as required for your project and delete sections not used>>

    - Standard shower pan(s) with fixed point drain

      - Model(s) shall be as follows:

        - <Insert shower base size and model number here>

    - Standard APTUM SERIES shower pan(s) with variable point drain

      - Model(s) shall be as follows:

        - <Insert shower base size and model number here>

    - Standard TRINCEA SERIES shower pan(s) with integral trench drain

      - Model(s) shall be as follows:

        - <Insert shower base size and model number here>

      - Trench drain shower pan(s) shall be supplied with removable stainless steel trench drain cover

        - Thickness to be 14 gauge

        - Finish to be #4

  - Custom Shower Pans/Shower Receptors

    - Custom shower pans shall be Meridian Solid Surface®, manufactured by Tower Industries, Inc., 2101 Ninth St. SW, Massillon, OH  44647. Phone 330.837.2216.

    - Custom shower pans shall be manufactured from solid cast polymer and have adjustable drain locations.

    - Shower Pans shall be a minimum of 9/16” thick at the thinnest point of the drain orifice and a minimum of 1-1/4” thick at the perimeter.

    - Shower pans shall be comprised entirely of Meridian Solid Surface®, with no voids or filler materials at underside of pan.

<< SPECIFIER NOTE:

<< Choose section v, vi, or vii as required for your project and delete sections not used>>

    - Custom BASIC SERIES shower pan(s)

      - Custom BASIC SERIES shower pans shall be supplied with a minimum 4” high water dam at three sides and a 2-1/2” high water dam at threshold side.

        - Water dams shall be a minimum of 1/2” thick

      - Custom BASIC SERIES shower pans shall have a textured, non-skid floor pitched toward the drain at a minimum 0.6° slope (1/8” per foot).

      - Floor texture for custom BASIC SERIES shower base shall be pebble-textured, Model MSB0000CB.

(Enter base size in lieu of the zeroes)

        - Shower pans with pebble-textured floor shall have a non-slip co-efficient rating of .30 or greater as registered by the ASTM F462 slip resistance test method.

      - Custom BASIC SERIES shower pans shall be supplied with raised independent solid surface threshold at one open side, Model MSBTD00RB

(Enter threshold length in lieu of the zeroes)

    - Custom ELITE SERIES shower pan(s)

      - Custom ELITE SERIES shower pans shall be supplied with a minimum 4” high, bi-level water dam at three sides and 3” high water dam at threshold side.

        - Water dams shall be a minimum of 1/2" thick at highest point

        - Lower ledge at bi-level water dams shall be a minimum of 3/4” wide

        - Water dam to be omitted at threshold side when using Independent Barrier-Free Threshold

      - Custom ELITE SERIES shower pans shall have a textured, non-skid floor pitched toward the drain at a minimum 0.6° slope (1/8” per foot). Floor texture for custom shower base shall be as follows:

        - Pebble-Textured, Model MSB0000CP

(Enter base size in lieu of the zeroes)

          - Shower pans with pebble-textured floor shall have a non-slip co-efficient rating of .30 or greater as registered by the ASTM F462 slip resistance test method.

      - Custom ELITE SERIES shower pans shall be supplied with integral solid surface threshold at one open side. Threshold type shall be as follows:

        - <Insert shower threshold type and model number here>

    - Custom TRINCEA SERIES trench drain shower pan(s)

      - Custom TRINCEA SERIES trench drain shower pan(s) shall be supplied with a minimum 4” high, bi-level water dam at three sides and no water dam at threshold side.

        - Water dams shall be a minimum of 1/2" thick at highest point

        - Lower ledge at bi-level water dams shall be a minimum of 3/4” wide

      - Custom TRINCEA SERIES trench drain shower pan(s) shall have a textured, non-skid floor pitched toward the drain at a minimum 0.6° slope (1/8” per foot).

      - Floor texture for custom TRINCEA SERIES trench-drain shower base shall be pebble-textured, Model MSB0000CPTF.

(Enter base size in lieu of the zeroes)

        - Shower pans with pebble-textured floor shall have a non-slip co-efficient rating of .30 or greater as registered by the ASTM F462 slip resistance test method.

      - Custom TRINCEA SERIES trench-drain shower pan(s) shall be supplied with Integral Accessible Solid Surface Threshold at one open side.

        - Integral Accessible Shower Threshold, Model MSBTI00ACC

(Enter threshold length in lieu of the zeroes)

  - Shower Wall Panels

    - Shower wall panels shall be Meridian Solid Surface®, manufactured by Tower Industries, Inc., 2101 Ninth St. SW, Massillon, OH  44647. Phone 330.837.2216.

    - Shower wall panels shall be manufactured from solid cast polymer.

    - Shower wall panel type shall be as follows:

      - 1/4” thickness, Model MSP140000

(Enter panel size in lieu of the zeroes)

    - Shower wall panels measuring 60” or less in width shall be supplied as single panels with no joints

    - Shower wall panels shall be manufactured and supplied at largest practical sizes so as to minimize seaming at the jobsite

    - Shower wall panels shall be trimmed and scribed at time of installation to ensure proper fit

  - Shower Partitions

    - Shower partitions shall be Meridian Solid Surface®, manufactured by Tower Industries, Inc., 2101 Ninth St. SW, Massillon, OH  44647. Phone 330.837.2216.

    - Shower partitions shall be manufactured from solid cast polymer.

    - Shower partitions type shall be as follows:

      - <Insert shower partition thickness and model number here>

    - Shower partitions shall be finished two sides with finished edges as required.

    - Shower partitions shall be manufactured and supplied at largest practical sizes so as to minimize seaming at the jobsite.

    - Shower partitions shall be trimmed and scribed at time of installation to ensure proper fit

  - Shower Trim Moldings

    - Shower trim moldings shall be Meridian Solid Surface®, manufactured by Tower Industries, Inc., 2101 Ninth St. SW, Massillon, OH  44647. Phone 330.837.2216.

    - Shower trim moldings shall be manufactured from solid cast polymer.

    - Shower trim molding type(s) shall be as follows:

      - <Insert shower trim type and model number here>

    - Shower trim moldings shall be supplied loose for trimming and scribing at time of installation

  - Shower Accessories

    - Shower accessories shall be Meridian Solid Surface®, manufactured by Tower Industries, Inc., 2101 Ninth St. SW, Massillon, OH  44647. Phone 330.837.2216.

    - Shower accessories shall be manufactured from solid cast polymer.

    - Shower accessory types(s) shall be as follows:

      - <Insert shower accessory type and model number here>

- Color

  - Shower Base & Threshold Color

    - To be selected from manufacturer’s core selection of solid colors

    - To be selected from manufacturer’s core selection of granite-patterned colors

  - Shower Wall Panel Color

    - To be selected from manufacturer’s core selection of solid colors

    - To be selected from manufacturer’s core selection of granite-patterned colors

  - Shower Accessory & Trim Molding Color

    - To be selected from manufacturer’s core selection of solid colors

    - To be selected from manufacturer’s core selection of granite-patterned colors

- Finish

  - All products to be supplied in Matte Finish

- Basic Limitations

  - Material designed for interior use only.

- Performance Characteristics

  - Comprehensive Analysis Data - Meridian Solid Surface®

| Test Type | Test Method | Meridian Results |
| --- | --- | --- |
| Hardness | ASTM D2583 | 60 - 62 |
| Tensile Strength | ASTM D638 | 5780 psi |
| Tensile Modulus | ASTM D638 | 1.2E+06 psi |
| Shower Receptor Load Test | Custom | 78,375 lbs |
| Gloss | NEMA LD 3-3.2 2005 | 5- 20 |
| Cleanability and Wear | CSA B45.5-11/IAPMO Z-124-2011 | PASS |
| Cigarette Test | CSA B45.5-11/IAPMO Z-124-2011 | PASS |
| Boiling Water Resistance | NEMA LD 3-3.5 2005 | No Effect |
| High Temp Resistance | NEMA LD 3-3.6 2005 | No Effect |
| Colorfastness Test | CSA B45.5-11/IAPMO Z-124-2011 | PASS |
| Stain Resistance Test | CSA B45.5-11/IAPMO Z-124-2011 | PASS (21) |
| Water Absorption | ASTM D570 | 0.052 |
| Flammability | ASTM E-84 | Class 1 |
| Impact Resistance – ¼” thick | NEMA LD 3-3.8 2005 | 86.5” |
| Impact Resistance – ½” thick | NEMA LD 3-3.8 2005 | 148” |
| Impact Resistance – ¾” thick | NEMA LD 3-3.8 2005 | 253” |
| % Cured | Differential Scanning Calorimetry | 100% |
| Bacteria Resistance | ASTM G22 | 2 |

PART 3 - EXECUTION

3.1 EXAMINATION

- Examine all substrates to receive solid surfacing for compliance with installation tolerances and other conditions affecting performance of work.

- Proceed with installation only after unsatisfactory conditions have been corrected.

3.2 INSTALLATION

- Install products plumb and level in locations indicated, using manufacturer’s recommended installation adhesives and materials, and in accordance with manufacturer’s recommended installation procedures.

  - Trim and scribe all shower wall panels and shower trim moldings to ensure proper fit.

  - Form field joints at wall panels (where required) using manufacturer’s recommended adhesive, with joints inconspicuous in finished work.

  - Cut and finish component edges with clean, sharp returns.

- Install grab bars and other accessories through solid surface products by drilling holes through solid surface at a larger diameter than the diameter of the fastener. Fastener (or threads of fastener) shall not come into contact with solid surface. Sand any chips surrounding penetration prior to inserting fastener.

- Coordinate plumbing installation with Division 22.

3.3 CLEANING

- Keep components clean during installation.

- Remove all adhesives and sealants from the product surfaces.

- After installation is complete, remove all excess adhesive, dirt and debris from the installation area.

3.4 PROTECTION

- Protect surfaces from damage until project is completed. Repair or replace damaged components which cannot be repaired to the Architect’s satisfaction.